Computer Architecture	Course: IT089IU

International University – VNU HCM 	Date:  March 2021

Dr. Le Hai Duong & Dr. Ly Tu Nga	Time: 6 hours

Laboratory Session 1

Introduction to Qtspim and Code composer Studio

Objectives
After completing this experiment, you will be able to:

-How to use QtSpim for simulation (Link) and Code Composer Studio for implementation (Link).

-How to understand, explain, and analysis the sample codes in simulation and reality.
 Activities
     -Sample codes.

-Demo Kit MSP430 (devices are supported by Instructor)

Materials Needed

-Read carefully: Assemblers_Linkers_and_the_SPIM_simulator.pdf, QtSpim-Tutorial.pdf, MIPS_ short.pdf, msp430g2553_doc2.pdf, MSP_doc1.pdf

# I. Introduction to SPIM (70pts)

- SPIM

Spim is a self-contained simulator that runs MIPS32 programs. It reads and executes assembly language programs written for this processor. Spim also provides a simple debugger and minimal set of operating system services. Spim does not execute binary (compiled) programs.

Download and install the newst version of Spim called QtSpim from: http://spimsimulator.sourceforge.net/

- QtSpim

QtSpim has three windows:

- Left Window: Registers

- Text Window: Code

- Data Window: Data, Stack, and Kernel data

Memory organization:

| 0x00400000 | Code |
| --- | --- |
| 0x10000000 – 0x10040000 | Data |
| 0x7fffeffc | Stack |
| 0x80000180 | Kernel code |
| 0x90000000 | Kernel data |

- First MIPS code (25pts)

- Download hello.s

- In QtSpim, File > Reinitialize and Load File to open hello.s

- Simulator > Run or press F5 to run the program

- The code will print “Hello, MIPS” on Console Window

Questions and Tasks:

  - The code was loaded into memory. Determine the memory address where the code resides.

In User Text Segment

The adr is 0040000

  - Using Single Step button or F10 to step through the code (execute the code one instruction at a time). In the Text Window, one instruction is highlighted after every step. Is this instruction the current executing instruction or the next instruction? How can you know that? Hint: there is a register indicates which instruction will be executed next.

  - The answer is the next instruction

  - Before

  - After

  - The string “Hello, MIPS\n” was loaded into memory. Determine its location in memory. Show the content of the memory segment which stores that string (in hex). Hint: look into Data Window.

  - Create an assembly code to print out your full name and student ID on separate lines.

- Load the program lab1_integer_double.s and run it in QtSpim. Remember to refresh register after each run. (25pts)

  - What is the output when you enter the integer 343523343532? Is the output correct? If not, explain what happened.

  - The answer is not correct because it is out of range integer values when it is doubled from that input.

  - What is the maximum and minimum input values that program can calculate correctly?

  - To calculate correctly, the output is integer value and the range of the input is from -1073741824 to +1073741824

  - Modify the program so that it prints newline after outputting the result. Save your assembly as  lab1_integer_double_01.s

  - Modify your program so that it print out the value of the first byte of the input. Test your program with the input 1463423. Save your program as lab1_integer_double_02.s

- Write an assembly that reads in 2 integers and prints out the result of addition and subtraction of those 2 integers. Save the assembly as  lab1_twointegers.s (20pts)

# II. Introduction to CCS (30pts)

+Download and install the newst version of Code Composer Studio from:

https://software-dl.ti.com/ccs/esd/documents/ccs_downloads.html

+ Create the new project by Alt+Shift+N

+ In Target, type msp430g2553 and in project name please type yourfullname_lab1, select Empty Project (with main.c)

+ In main.c, type the sample code of Table 1

Table 1. control LED under controlled Loop in MSP430

| No. | Test (Sample Code) | Comments/Results/Functions |
| --- | --- | --- |
| 1. 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 | #include <msp430.h> #define LED1 BIT0 #define LED2 BIT6 #define DELAYLOOPS 32000 /* main.c */ void main(void) { int LoopCtr; WDTCTL = WDTPW ¦ WDTHOLD; P1DIR =LED1¦LED2; P1OUT =LED1¦LED2; while(1){ for(LoopCtr=0;LoopCtr<DELAYLOOPS;++LoopCtr) { } P1OUT ^=LED1¦LED2; } } | Macro LED1 (assigned P1.0 or BIT0) Macro LED2 (assigned P1.6 or BIT6) Stop watchdog timer. Setup LED1 and LED2 to direction output and turn on empty delay loop toggle LEDs |

+ Click build “debug” and observe the console window: Build finished

+ Click Run Debug and select Run in background

Step 1:  Evaluation memory allocation: click View> Memory Allocation

Fill in the percentage of RAM allocation: ____15%__________ and files:

Fill in the percentage of FLASH allocation: ______0%________ and files:

Step 2: Not run, click lnk_msp430g2253.cmd to fill in these answers.

Summary some sentences of SECTIONS (Specify the sections allocation into memory: lines 88-109

Step 3: open test.map and fill in these addresses following:

address: 0000c02a

In Section Allocation MAP:

-text: 0    0000c000    00000050

-stack: 0    000003b0    00000050

-WDT: 0    0000fff4    00000002

-Port 1: 0    0000ffe4    00000002

-Reset: 0    0000fffe    00000002

Memory Configuration: INT02: 0000ffe4

Global symbols: sorted alphabetically by Name:

-P1IN: 00000020

-P1OUT: 00000021

- main: 0000c000

-WDTCL:_ 00000120

-LoopCtr: _____29285_____

How many symbols? 125

Step 4: The Value of registers:

P1IN: before 00000020 and after 00000020

P1OUT: before 00000021 and after 00000021

LoopCtr: before _____29285_____ and after __913_________

Hint: you can use Expression or open the Core Registers tab (Port_1_2) to find the solution

Explain the values of P1IN and P1OUT registers in Port_1_2:  _______no change_________________________________________________________________________________________________________________________________________

Step 5: When running and pausing, observe the Core Registers in Registers tab and please write down comments about the transition of these registers in there.

Core Registers PC value change from 0xC000 to 0xC01A

Core Registers SR value change from 0x0000 to 0x0004

Core Registers R15 value change from 0x7265 to 0x0391

Step 6: how to use breakpoint in CCS : Alt+Shift+Q,B

________________________________________________________________________________________________________________________________________________

Step 7:  When running and pausing, click View and open Disassenbly window, write down these instructions of sample code:

Step 8: Based on the step 7, please explain the process of sample code under Computer Architecture structure

Step 9: Change the time DELAYLOOPS to the period of LED1 is twice that of LED2.

#include <msp430.h>

#define LED1 BIT0

#define LED2 BIT6

#define DELAYLOOPS 32000

/* main.c  */

void main(void)

{

int LoopCtr;

WDTCTL = WDTPW | WDTHOLD;

P1DIR =LED1|LED2;

P1OUT =LED1|LED2;

while(1){

for(LoopCtr=0;LoopCtr*2<DELAYLOOPS;++LoopCtr)

{

}

P1OUT ^=LED1|LED2;

}}

Your code is changed (after successfully checked)

Explain the registers, address, memory, etc which will be changed to fulfill the new condition.  ______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

# Reference:

- https://en.wikibooks.org/wiki/MIPS_Assembly/Pseudoinstructions

- https://courses.missouristate.edu/KenVollmar/MARS/Help/SyscallHelp.html

- https://www.assemblylanguagetuts.com/mips-assembly-programming-tutorials/#MIPS_Data_Types

- https://en.wikibooks.org/wiki/MIPS_Assembly/Arithmetic_Instructions

- https://gab.wallawalla.edu/~curt.nelson/cptr280/lecture/mips%20arithmetic%20instructions.pdf